Task1:

Description:

- RB0, RB1, RB2 – Configured as Inputs

- RD0,1,2,5,67 – Configured as Outputs

- As Long as RB0 is pressed it is stuck in While loop toggling the RD0 and RD1  every 1 Second

- As Long as RB1 is pressed it is stuck in While loop toggling the RD7 and RD6 every 1 Second

- As Long as RB2 is pressed it is stuck in While loop toggling the RD0,RD1,RD2,RD5,RD6,RD7 every 1 Second

Schematic Diagram

Code:

void main() {

TRISB = 0x07;   //RB0, RB1, RB2 as Input Pins

TRISD = 0x00;   //All the Pins in Port configured as Output

PORTD = 0x00;

while(1)

{

PORTD =0x00;

while(!(PORTB & 0x1) )

{

PORTD ^= 0x3;

Delay_ms(1000);

}

while(!(PORTB & 0x2) )

{

PORTD ^= 0xC0;

Delay_ms(1000);

}

while(!(PORTB & 0x4) )

{

PORTD ^= 0xE7;

Delay_ms(1000);

}

}

}

PICSIMLAB Output

Task2:

Description:

- AN0 is configured as Analog Input Pin, and Interanl Vref+ (5v) and VSS (0V) is used for converting the input voltage

- Max 5V – will give ADC output value 1023

- We convert max 5 volt value to 150 --> i.e 15.0V

- Whenever the voltage drops below 10.5 – Low Voltage is Displayed

- Whenever the voltage raises above 10.5  and below 13.5 –Normal Voltage is Displayed

- Whenever the voltage raises above 13.5 High Voltage is Displayed

Schematic:

Code:

void Init(void);

void LCD_Command(unsigned char);

void LCD_Data(unsigned char);

void LCDOutput(unsigned int);

void Delay(unsigned int);

unsigned char k[10],x;

unsigned char n,m;

unsigned int hivalue,lovalue,adcv;

long value;

char Lowstring[] = "Low  Voltage";

char Highstring[] = "High Voltage";

char Normstring[] = "Normal Voltage";

char *ptr;

void update_lcd(unsigned int num)

{

LCD_Command(0x80);               //Initialize cursor to first Position

LCDOutput(num);

if(num > 135)

{

ptr = (char *)Highstring;

}

else if(num < 105)

{

ptr = (char *)Lowstring;

}

else

{

ptr = (char *)Normstring;

}

LCD_Command(0xC0);

while(*ptr != '\0')

LCD_Data(*ptr++);

Delay(100);

}

void main()

{

Init();

ADCON0=0x00;                            // sampling freq=osc_freq/2,ADC off initially

ADCON0=0x81;                            //configure the A/D control registers

ADCON1=0x8E;

while(1)

{

ADCON0|=0X04;                           //start ADC conversion

while(ADCON0&0X04);                     //wait for conversion to complete

lovalue=ADRESL;                         //read the low 8 bit value

hivalue=ADRESH;                         //read the upper 8 bit value

value=((unsigned int)hivalue<<8)+(unsigned int)lovalue;

adcv = (value*150)/1023 ;

update_lcd(adcv);

}

}

/*end main program*/

void Init(void)

{

TRISD = 0x00;                   //Initialize the PORTD as output

TRISC = 0x00;                   //Initialise the PORT C as output

TRISA = 0x01;

LCD_Command(0x38);              //Initialize the 2 lines and 5*7 Matrix LCD

Delay(100);

LCD_Command(0x38);

Delay(100);

LCD_Command(0x38);

Delay(100);

LCD_Command(0x38);

Delay(100);

LCD_Command(0x06);             //Increment cursor (shift cursor to right)

Delay(100);

LCD_Command(0x0C);             //Display on,cursor off

Delay(100);

LCD_Command(0x01);             //clear display screen

Delay(100);

}

/*define the output function*/

/*BCD conversion*/

void LCDOutput(unsigned int num)

{

unsigned int j;

unsigned int i;

unsigned int tdata;

tdata = num   ;

if(tdata == 0)

{

LCD_Data(0x30);                     //assign formal argument to other variable

LCD_Data(0x30);

LCD_Data(0x2E);

LCD_Data(0x30);

LCD_Data('V');

}

else

{

j=0;

while (tdata != 0)

{

i = tdata - (tdata / 10) * 10;

k[j] = i+0x30;

tdata = tdata / 10;

j++;

}

k[j] = '\0';

//LCD_Data(k[3]);

LCD_Data(k[2]);

LCD_Data(k[1]);

LCD_Data(0x2E);

LCD_Data(k[0]);

LCD_Data('V');

}

}

void LCD_Command(unsigned char i)

{

PORTC&=~0x04;               // RS=0

PORTD=i;

PORTC |=0x02;               // RS=0,R/W=0,EN=1

PORTC&= ~0x02;              // RS=0,R/W=0,EN=0

Delay(100);

}

void LCD_Data(char i)

{

PORTC|=0x04;              //RS=1

PORTD=i;                  //Assign the value to PORTD to display

PORTC|=0x02;              // RS=1,R/W=0,EN=1

PORTC&=~0x02;             // RS=1,R/W=0,EN=0

Delay(100);

}

void Delay(unsigned int DelayCount)

{

while(--DelayCount);

}

PICSIMLAB - Output

Task 3: 7 Segment Display with Transistor Multiplexer

Description:

- PORT D is configured as common inputs for the 4 7 Segment Display

- PORTA is used as Multiplexer used to select particular 7 Segment Display

Code – PICSIMLAB

void main()

{

unsigned int i,j;

TRISD = 0x00;   //7 Segment Display output

TRISA = 0x00;   //Transistor Multiplexer output

while(1)

{

for(i =0; i < 10; i++)

{

PORTD = digits[i];

for(j= 0; j <4 ; j++)

{

PORTA = 0x20 >> j;

Delay_ms(30);

}

Delay_ms(50);

}

}

}

PICSIMLAB - Output

Schematic: with 3 Segment Display

Code:

//RA2-RA5 - Transistor Mux

//RD0-RD7 - LED OUTPUT

#define D0 0x3F

#define D1 0x06

#define D2 0x5B

#define D3 0x4F

#define D4 0x66

#define D5 0x6D

#define D6 0x7D

#define D7 0x07

#define D8 0x7F

#define D9 0x6F

unsigned int digits[10] = {D0, D1, D2, D3, D4, D5, D6, D7, D8, D9};

void main()

{

unsigned int i,j;

TRISC = 0x00;   //7 Segment Display output

TRISD = 0x00;   //Transistor Multiplexer output

while(1)

{

for(i =0; i < 10; i++)

{

PORTC = digits[i];

for(j= 0; j <3 ; j++)

{

PORTD = 0x4 >> j;

Delay_ms(30);

}

}

}

}

Proteus Output: